| Date Emailed 10/25/2024 |  |  |  |  | # REQUEST FOR BID | RFB No. 924-2 |  |  |  |  | Page No. 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| BID MUST BE SUBMITTED ON THIS FORM. FORM MUST BE SIGNED AND RETURNED WITH ANY SUPPORTING DOCUMENTATION. THIS IS NOT AN ORDER |  |  |  |  | State of Iowa ## IOWA STATE INDUSTRIES 406 North High Street Anamosa, Iowa 52205-0430 All inquiries should be directed to ### RUTHIE MOSSER, PURCHASING AGENT Phone 319-462-3706 Fax No. 319-462-2158 ruthanne.mosser@iowa.gov | Complete this form per enclosed instructions and email to iowaprisonindbids@iowa.gov 11/07/2024 |  |  |  |  |  |  |
| Qty. | Units | Description of Item |  |  |  |  | Item Price |  | Amount |  |  |  |
| Pour concrete for a shop floor with the following specifications: Slabs-on-Grade: Normal-weight concrete. Design based on 10,000-Pound Maximum - 60'x160' 9,600 SF 1. Minimum Compressive Strength: 4000 psi mix design 2. Concrete thickness 6” PCC with #4 rebar @16” OC each way (Minimum concrete coverage on rebar of 2” and must not lay on the gravel). 3. Subgrade – Excavation depth as needed onsite to provide 6” compacted granular fill and the 6” PCC at interior walls and align flush with build pad aprons, etc. Concrete tie-in with #4 rebar and epoxy to existing aprons 2’ OC. Also, include 6 mil vapor barriers under all slab space. (we suggest scrapping existing gravel off to dirt for re-use of gravel) 4. Broom finish 5. Maximum W/C Ratio: 0.50. 6. Slump Limit: Up to 6 inches for concrete with verified slump of 2 to 4 inches before adding waterreducing admixture or plasticizing admixture. 7. Air Content for Exterior Slabs: 6 percent, plus or minus 1.5 percent at point of delivery. 8. Saw joints area shall be no larger than 200SF spacing 9. Cold Joints detailed at each pour(s). Expansion joint at all cold joints and warehouse interior walls/slab on grade. A site visit can be coordinated with Chad Squires, 307 South 60th Ave Newton, IA Chad.squires@iowa.gov Phone: 641-841-7721 Estimated time of completion ___________ Needs to be completed prior to a hard frost. TOTAL COST: ______________________ |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | IOWA PRISON INDUSTRIES RESERVES THE RIGHT TO REJECT ANY AND ALL BIDS |  |  |  |  |  |  |  |  |
| It is not the intent of the office of purchasing to rule out competition in the specifications so listed. Bids from other manufacturer’s standard unit of production will be accepted; However, any deviations must be noted. Preference shall be given to purchasing Iowa products and purchases from Iowa-based businesses if the Iowa-based business bids submitted are comparable in price to bids submitted by out-of-state businesses and otherwise meet the required specifications. By virtue of statutory authority, a preference will be given to products and provisions grown and coal produced within the state of Iowa. Preference application: Tied responses to solicitations, regardless of the type of solicitation, are decided in favor of Iowa products and Iowa-based businesses per 11 IAC 105.5(1)-(2), 105.12(4).. If the laws of another state mandate a percentage preference for businesses or products from that state and the effect of the preference is that bids from Iowa businesses or products that are otherwise low and responsive are not selected in the other state, the same percentage preference shall be given to Iowa businesses and products when businesses or products from that other state are bid to supply Iowa requirements.. |  |

|  |  |  | Days. | Terms |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | F.O.B. | NEWTON, IA |  |
| Authorized Signature |  |  |  | #### SPECIAL INSTRUCTIONS TO BIDDERS 1. THIS IS NOT AN ORDER. - Each item must be priced separately. - All prices should be on a delivered basis – if not, bidder must show F.O.B. point. - Show unit of measure i.e. – ‘Doz’ or ‘each’. - All items in this form must be filled in. - Bidder agrees that if awarded a contract to supply any part of the above listed material, bidder will not engage in any discriminatory employment practices based upon race, color, creed, religion or national origin and that they will in all contracts comply with all statutes of the State of Iowa against discrimination. Failure to do so could be deemed a breach of contract. |  |  |
| Email Address |  |  |  |  |  |  |
| Firm Name |  |  |  |  |  |  |
| Address |  |  |  |  |  |  |
| City/State/Zip Code |  |  |  |  |  |  |
| Phone No. |  | Fax No. |  |  |  |  |